Exercise

The purpose of this exercise is:

- To investigate the cause of offset in a P-only controller

- To investigate the cause of oscillations in a PI controller

## Open Loop Response

First Order Plus Dead time (FOPDT) model:

τpdydt=-y+Kpu(t-θp)

Where:

- Kp = 2

- τp = 200

- θp = 0

- Simulate the behavior for making a step change in manual mode from 0 to 10 (and back). Explain what happens in terms of oscillations or a smooth response.

Solution

There are no oscillations, the response is smooth.

## P-only control

Simulate the behavior for using a P-only controller with:

- Kc=2

- Kc=0.5

Implement a set point change from 0 to 10 and back in automatic mode (closed-loop). Include a plot of the error between the set point (SP) and process variable (PV). What happens with increased Kc in terms of offset and oscillation?

Solution

Kc = 2

Kc = 0.5

With an increased Kc, the offset is smaller and the final values is reached sooner.

## PI control

Configure the controller to add an integral term in addition to the proportional control with:

- Kc = 2

Simulate the PI controller response with integral reset times:

- τI = 200, 100, 10

Include a plot of the integral of the error between the set point (SP) and process variable (PV) with anti-reset windup. Explain what happens and why.

Solution

- Smaller values for the controller's integral time constant produce an increased oscillatory behavior on the output.

- The anti-reset windup prevents and increasing error accumulation when an upper or lower saturation occurs.

τI = 200

τI = 100

τI = 10

τI = 10 and no anti-windup:

## Open Loop Response with Dead Time

Add dead time θp=100 as an input delay. Simulate the behavior for making a step change in manual mode from 0 to 10 (and back). Explain what happens in terms of oscillations.

I see no oscillations in the open loop response, only the expected lag.

## P-only Control with Dead Time

With the dead time, simulate the response of a P-only controller with:

- Kc=2

- Kc=0.5

Implement a set point change from 0 to 10 and back in automatic mode (closed-loop). Include a plot of the error between the set point (SP) and process variable (PV). What happens with increased Kc in terms of offset and oscillation?

Solution

With Kc = 2:

With Kc = 0.5:

With an increased Kc, the offset diminishes but the oscillations increase.

## PI Control with Dead Time

Simulate the response of a PI controller with τI=200. Include a plot of the integral of the error between the set point (SP) and process variable (PV) with anti-reset windup. Explain what happens and why. Explain the results.

Solution

With, Kp = 2, τp = 200, θp = 100 and Kc = 2, τI = 200:

Having a dead time really complicates matters as it adds oscillations. When there was no dead -time and a PI control was used, the response converged without oscillations.

## Summary Questions

- Based on the observations in manual mode, is the process stable or unstable?

- Why does P-only control have persistent offset?

- What is the effect of dead time on P-only and PI control?

- If the process is stable, why can the control system make it unstable?

Solution

- The problem seems to be stable as the output is bounded.

- P-only control has persistent offset only when the process is non-integrative.

- Dead time produces oscillations when in closed-loop.

- I don’t know.